L298N Motor Control

Setup & Usage Guide

Web UI · CLI · Autopilot · SSH Tunnel

# Overview

There are three Python scripts for controlling a DC motor via an L298N driver board connected to a Raspberry Pi. Choose the one that fits your situation:

All scripts use gpiozero, the current recommended GPIO library for Raspberry Pi OS. gpiozero handles pin setup, PWM and cleanup automatically. The older RPi.GPIO library is no longer actively developed and requires manual pin configuration, PWM setup and GPIO.cleanup() calls — gpiozero takes care of all of this for you.

| Script | What it does |
| --- | --- |
| L298N_WebUI_Autopilot.py | Full browser-based dashboard with animated panel, speed slider, autopilot sequence and safety watchdog. |
| L298N_CLI_Autopilot.py | Terminal command-line control. No browser needed. Same autopilot and boost logic. Good for headless setups. |
| L298N_WebDemo.py | Simpler web demo — forward / stop / backward only, no autopilot. Good starting point for students. |

# Wiring

Connect the Raspberry Pi GPIO pins to the L298N as follows. Remove the ENA jumper from the L298N board — this is required for PWM speed control.

| Raspberry Pi | L298N |
| --- | --- |
| GPIO 17 | IN1 (direction) |
| GPIO 27 | IN2 (direction) |
| GPIO 18 | ENA (PWM speed — remove jumper first) |
| GND | GND (shared ground with motor supply) |
| — | VCC → 9 V battery + |
| — | OUT1 + OUT2 → Motor terminals |

Do not power the motor from the Raspberry Pi 5 V pin. The motor needs its own supply connected to the L298N VCC terminal.

# One-Time Setup (run once on the Raspberry Pi)

Open a terminal on the Raspberry Pi and run:

pip install flask gpiozero

That is the only dependency. No other packages are needed.

# Web UI — L298N_WebUI_Autopilot.py

## Step 1 — Start the server on the Raspberry Pi

In a terminal on the Pi, run:

python L298N_WebUI_Autopilot.py

You will see no output until a browser connects. Leave this terminal open — closing it stops the server.

## Step 2 — Open the SSH tunnel on your PC

On your classroom PC, open a terminal (PowerShell or Command Prompt on Windows, Terminal on Mac/Linux) and run:

ssh -L 5000:localhost:5000 pi@<raspberry-pi-ip>

Replace <raspberry-pi-ip> with the actual IP address of your Pi, for example: ssh -L 5000:localhost:5000 pi@192.168.1.42

Enter your Pi password when prompted. Keep this terminal open — the tunnel closes if you close it.

## Step 3 — Open the dashboard

In any browser on your PC, go to:

http://localhost:5000

The animated control panel will appear. You can now control the motor from your browser.

## Controls

- Forward / Backward buttons — drive the motor in that direction at the current slider speed.

- STOP button — immediately stops the motor and cancels the autopilot.

- Speed slider — sets the display speed. 1 % on the slider = 10 % motor power internally, so the motor starts reliably at low settings.

- Autopilot button — starts a repeating sequence: ramp forward 10 % → 100 %, pause, ramp backward 10 % → 100 %, pause, repeat. Click again to stop.

- Arrow UpKeyboard shortcuts:

Arrow Up = Forward   Arrow Down = Backward   Space or Esc = STOP

## Safety features

- Watchdog thread — if no command arrives for 2 seconds (network drop, PC sleeps), the motor stops automatically.

- Heartbeat — the browser sends a signal every 500 ms. If you close the browser tab the heartbeat stops and the watchdog triggers within 2 seconds.

- SSH tunnel only — the server listens on localhost only, so it is not accessible on the wider school network without the tunnel.

# CLI — L298N_CLI_Autopilot.py

## Start

On the Raspberry Pi (directly or over SSH), run:

python L298N_CLI_Autopilot.py

A help menu prints automatically. You are now in the control prompt.

## Commands

| Command | What it does |
| --- | --- |
| f 50 | Forward at 50 % display speed |
| f 30 | Forward at 30 % (motor runs at ~37 % internally) |
| b 70 | Backward at 70 % |
| s or stop | Stop the motor immediately |
| a or auto | Start autopilot (ramp forward, pause, ramp backward, pause, repeat) |
| x | Stop the autopilot |
| status | Print current direction and speed |
| h or help | Show the command reference |
| q or quit | Stop motor and exit |

## Speed note

The display speed you type (1–100) is mapped to internal motor power automatically:

- 1 % display  →  10 % motor power (minimum that starts reliably)

- 50 % display  →  55 % motor power

- 100 % display  →  100 % motor power

When starting from stopped or changing direction at low speed, a 100 % boost pulse fires for 180 ms to overcome motor friction, then the speed drops to the set level.

# Troubleshooting

| Problem | Fix |
| --- | --- |
| Motor does not move at all | Check wiring. Check that the ENA jumper is removed. Check that the external motor supply is connected. |
| Motor moves but speed control fails | Confirm GPIO 18 is connected to ENA and USE_ENABLE_PIN = True in the script. |
| Browser cannot reach the dashboard | Make sure the SSH tunnel command is running in an open terminal. Check the Pi IP address. |
| Motor starts then stops after 2 s | The watchdog triggered. Make sure the browser tab is open and the heartbeat is reaching the Pi. Check your network. |
| Motor does not start at low speed | This is normal below about 10 % real motor power. The boost pulse handles it automatically. Use at least 1 % on the slider. |
| ImportError: No module named flask | Run: pip install flask gpiozero on the Raspberry Pi. |

# Quick Reference Card

| Task | Action |
| --- | --- |
| Start Web UI server | python L298N_WebUI_Autopilot.py (on the Pi) |
| Open SSH tunnel (PC) | ssh -L 5000:localhost:5000 pi@<pi-ip> |
| Open dashboard (PC browser) | http://localhost:5000 |
| Start CLI control | python L298N_CLI_Autopilot.py (on the Pi) |
| Install dependencies | pip install flask gpiozero (once on the Pi) |
| Emergency stop (Web UI) | Press STOP button or hit Space / Esc |
| Emergency stop (CLI) | Type s and press Enter |